# Project 2: Reactive Robotics on a Physical TurtleBot 4

# Change Documentation

Team: Sadia Tasnim, Thomas Kaseca, Kiara Nelson

CS 4023/5023 Introduction to Intelligent Robotics

Code: https://github.com/seepaa/IRP1/tree/project-2

Here are the changes we made to our code and launch files going from Project 1 (Gazebo simulation) to Project 2 (the real TurtleBot 4 in REPF B4), and why we made each one.

## Launch files

We kept simulation.launch.py from Project 1 unchanged for the simulation and wrote a new launch file, robot.launch.py, for the real robot. Compared to the Project 1 launch file:

- We took out Gazebo, the /clock bridge, spawning the robot, the Gazebo model paths, and the world and start-position options. The real robot runs its own drivers and already publishes /scan, /odom, /hazard_detection and /tf, so there is nothing left to simulate or spawn.

- We set use_sim_time to false. Without Gazebo nothing publishes /clock, so with sim time our clocks would stay at zero. The 5-second teleop timeout would never run out, and one key press would keep control forever.

- The launch file now turns the lidar motor on when it starts and off when we press Ctrl+C. The real lidar doesn’t spin until it is told to, and the lab rules say it has to be off before the robot goes back on its dock.

- We kept the step that turns off the robot base’s own bump reflex. The base turns it back on every time it reboots, and it would fight our halt behavior.

- The mapper is now optional and off by default (mapper:=false), since Project 2 doesn’t need a map.

- We added forward_speed, turn_speed and halt_on_cliff as launch options so we could tune the robot in the lab without editing code.

## Settings we changed

| Setting | Project 1 | Project 2 | Why we changed it |
| --- | --- | --- | --- |
| forward_speed | 0.2 m/s | 0.1 m/s | The OU lab guide recommends 0.1, and it is safer in a shared room |
| turn_speed | 0.8 rad/s | 0.5 rad/s | Less overshoot when turning on the real wheels |
| heading_tolerance_deg | 3 | 5 | Real odometry is noisier; a bigger tolerance stops the robot from wobbling back and forth around the target heading |
| min_valid_range | (new) | 0.15 m | So the lidar ignores the robot’s own tower posts |

## Controller (reactive_node.py)

- /odom is now read with best-effort (sensor data) settings. A reliable subscriber gets nothing from a best-effort publisher, so the controller would never get its first position and would just sit there with no error.

- In Project 1, if the controller didn’t know yet which way the lidar was mounted, it assumed it faced forward. The real TurtleBot 4’s lidar is mounted rotated, so that guess would mix up front and sides. Now it ignores the lidar until it knows the mounting.

- The robot now waits until it has both odometry and a usable lidar scan before it moves. If the lidar isn’t spinning, it stays still instead of driving blind.

- Halt now also stops for cliffs and wheel drops, not just bumps (halt_on_cliff, on by default), since those can damage the real robot.

- Halt now lets go on its own if hazard messages stop coming for 0.5 s (hazard_timeout). Over Wi-Fi a “released” message can get lost, and the robot would otherwise stay stopped forever.

- min_valid_range was added (see the table above).

## Mapper (occupancy_mapper.py)

- /odom is now read with best-effort settings, for the same reason as the controller.

## Package and README

- package.xml: we added std_srvs, which the lidar on/off commands need, and marked the Gazebo packages as only needed for the simulation. We also updated the package descriptions in package.xml and setup.py.

- README: we added the Project 2 steps (checking the robot over SSH, running robot-setup.sh in every terminal, launching, teleop, shutting down and docking). The Project 1 ROS_DOMAIN_ID=42 setup keeps a terminal from seeing the robot, so it is now only in the simulation section.

## Team Contributions:

Kiara Nelson adapted our Project 1 code and launch file for the physical TurtleBot 4. Thomas Kaseca handled launching the project on the robot. Sadia Tasnim did the robot testing and wrote the Project 2 documentation.