# Project 2: Reactive Robotics on a Physical TurtleBot 4

# Launch File Documentation

CS 4023/5023 Introduction to Intelligent Robotics

Team: Kiara Nelson, Sadia Tasnim, Thomas Kaseca

Repo: https://github.com/seepaa/IRP1/tree/project-2

## Overview

For Project 2 we wrote a new launch file, robot.launch.py, to run our controller on a real TurtleBot 4 in REPF B4. In Project 1 our launch file had to start Gazebo, load our world and spawn the robot. This time the robot already does most of that work itself. Its own computer runs the drivers for the lidar, wheels and bumpers, so our launch file just starts our code on the lab computer and takes care of a few things the real robot needs.

## What the launch file does

When we run it, the launch file does four things:

- It turns on the lidar motor. On the real robot the lidar does not start spinning by itself, so without this the robot cannot see anything.

- After two seconds, it turns off the robot base’s built-in bump reflex. By default the base backs away whenever it bumps into something, which fights with our own halt behavior.

- It starts our controller, which picks between the six behaviors (halt, keyboard, escape, avoid, random turn, forward). If we ask for it, it also starts our mapper, but Project 2 does not need a map, so that is off by default.

- When we stop it with Ctrl+C, it also stops the lidar, since we have to turn the lidar off before docking the robot.

We also made the main settings into launch options so they can be changed without editing any code: driving speed (forward_speed, 0.1 m/s), turning speed (turn_speed, 0.5 rad/s), whether to stop for cliffs as well as bumps (halt_on_cliff, on), whether to run the mapper (mapper, off), plus switches for the lidar, the reflexes and the message type (start_lidar, disable_reflexes, stamped).

## How to run it on the CSN Linux machines

1. One-time setup:

mkdir -p ~/ros2_ws/src && cd ~/ros2_ws/src
git clone -b project-2 https://github.com/seepaa/IRP1
cd ~/ros2_ws && source /opt/ros/jazzy/setup.bash && colcon build --packages-select proj1

2. Carry the robot by its base, put it on the floor, and log into it from a terminal:

ssh student@<robotName>.cs.nor.ou.edu   # password: student
ros2 topic list                          # should list /scan, /tf and /odom

If those topics are missing, run turtlebot4-daemon-restart, wait for the chime and check again. If the robot is on its dock, undock it:

ros2 action send_goal /undock irobot_create_msgs/action/Undock "{}"

3. In every terminal on the lab computer, run robot-setup.sh, type the robot’s name and run the lines it prints, in order. Then:

source /opt/ros/jazzy/setup.bash
source ~/ros2_ws/install/setup.bash

Don’t use the ROS_DOMAIN_ID=42 setup from Project 1 here, or the terminal won’t be able to see the robot.

4. In the first terminal, start everything:

ros2 launch proj1 robot.launch.py

5. To drive with the keyboard, open a second terminal (after step 3) and run the command below. Each key press takes control for 5 seconds. We turned the speeds down to about 0.1 m/s and 0.2 rad/s with the z and c keys.

ros2 run teleop_twist_keyboard teleop_twist_keyboard \
  --ros-args -p stamped:=true -r cmd_vel:=/teleop_cmd

6. When finished, press Ctrl+C in the first terminal (this also stops the lidar) and put the robot back on its dock.