# Project 1: Reactive Robotics using ROS 2 and Gazebo

# Robot Code Documentation

Team: Thomas Kaseca, Kiara Nelson, Sadia Tasnim.

CS 4023/5023 Introduction to Intelligent Robotics

## Overview and architecture

Our robot runs on two ROS 2 Python nodes that work side by side and only share sensor data. SubsumptionController (reactive_node.py) decides how the robot moves. OccupancyMapper (occupancy_mapper.py) builds the map and has no say in the driving.

The controller is a subsumption-style priority architecture. We rank the six behaviors from highest to lowest priority. Ten times a second the controller checks them in order, and the first one that wants control sets the wheel command. So a higher behavior always overrides the ones below it.

## Why this architecture

We picked this because the assignment already gives a ranked list of behaviors, and a ranked list turns into code almost directly. It also lets us read and test each behavior by itself.

It follows the idea of Brooks' subsumption and the pseudo-subsumption of Jones et al., not Brooks' exact wiring. His layers are separate machines joined by suppression and inhibition wires. Ours are six methods in one class, and a loop stops at the first one that returns a command. A behavior with nothing to say returns None, and control drops to the next one.

## How the behaviors are encoded

Each required behavior is one clearly named method, listed here from highest to lowest priority. Mapping runs in its own node.

| # | Behavior | Method | Wins control when | Output |
| --- | --- | --- | --- | --- |
| 1 | Halt | halt() | Any bumper reports a bump | Zero velocity |
| 2 | Teleop | teleop() | A keyboard command arrived in the last 5 s | The keyboard velocity |
| 3 | Escape | escape() | Obstacles in both front arcs within 1 ft, and the two distances differ by under 0.15 m | In-place turn of 180° ± 30°, run to completion |
| 4 | Avoid | avoid() | An obstacle within 1 ft in either front arc | Spin away from the closer side, only while it is there |
| 5 | Random turn | random_turn() | 1 ft of forward travel since the last turn | In-place turn drawn uniformly from ±15° |
| 6 | Forward | forward() | Always | 0.2 m/s straight ahead |
| – | Mapping | OccupancyMapper node | Always, in parallel | /map occupancy grid |

Escape is the fixed action pattern: it saves a target heading and keeps turning until the robot is within 3° of it, even if the obstacle is gone. Random turn uses the same latched-turn mechanism so a chosen turn finishes once it starts. Avoid is a true reflex: it saves nothing and stops as soon as the obstacle leaves the 1 ft zone. The launch file turns off the Create 3's own bump reflexes, so only our halt behavior reacts to bumps.

Where the architecture shows in the code. The behaviors tuple fixes the priority order. control_loop() stops at the first command that isn't None, so higher behaviors override lower ones. Each behavior is its own method, avoid() is recomputed every cycle, and turn_target and turn_owner keep escape and random turns running. The controller never reads /map.

## Data structures

- Behavior list. behaviors is a tuple of (name, bound method) pairs in priority order. The ranking is data, so changing it means reordering the tuple.

- Sensor state. bumper_hit (bool), front_left_dist and front_right_dist (closest lidar range in each 30° front arc, in meters), and scan_yaw_offset (the lidar's cached rotation relative to the robot).

- Odometry state. current_yaw, the last x and y, and forward_travel, the meters driven forward since the last random turn.

- Teleop state. teleop_twist and teleop_time, the last keyboard command and when it arrived.

- Turn latch. turn_target (a heading in radians) and turn_owner ('escape', 'random_turn' or None) store the turn in progress. This is what lets escape and random turn finish once started.

- Map grid. The mapper keeps two NumPy arrays of 400 × 400 cells (20 m × 20 m at 5 cm per cell): log_odds (float32) and seen (bool). It publishes them as a nav_msgs/OccupancyGrid, using −1 for unknown and 0–100 for occupancy.

## Algorithms

Control loop. A 10 Hz timer waits for the first odometry message, then goes through the behaviors in priority order and publishes the first command it gets. It logs a message whenever the winning behavior changes.

Halt and keyboard. Every message on /hazard_detection sets bumper_hit if any detection is a bump, and halt() then returns zero velocity. The keyboard node only publishes once per key press, so teleop() holds the last command for 5 s before the robot goes back to driving itself.

Finding obstacles. For each lidar beam we convert its angle to the robot's frame, using the lidar's mounting rotation from TF. Beams from 0° to +30° make up the left arc, and beams from −30° to 0° make up the right arc. We keep the closest valid range in each arc.

Escape or avoid. If both arcs read under 1 ft and differ by less than 0.15 m, we call the obstacle symmetric and escape takes over. Any other obstacle within 1 ft is asymmetric, and avoid spins at 0.8 rad/s away from the closer arc.

Turning. start_turn() saves a target heading: the current yaw plus the chosen angle, wrapped to [−π, π]. continue_turn() spins toward it the short way until the error is under 3°. If avoid has to act, it throws away any pending random turn, since that turn is out of date.

Forward distance. Each odometry update is projected onto the robot's heading. Only forward motion counts toward the 1 ft, so spinning or backing up adds nothing. The counter resets at every random turn.

Mapping. Each scan is placed in the odometry frame using the robot pose and the lidar's TF mount. Each beam is sampled every half cell. Cells it passes through get −0.4 log-odds (more likely free), and the cell where it ends gets +0.85 (more likely occupied). A cell changes at most once per scan, and values are clamped to ±5 so a cell can still change its mind. Once a second the log-odds become probabilities and the grid is published on /map with transient-local QoS, so RViz can join late.

## Design decisions and team credits

Robustness. The controller reacts only to what the lidar sees now, so it doesn't depend on the world's size or shape. The lidar's mounting rotation comes from TF, with a fallback from Gazebo's scoped sensor name to the link name, so the front arcs stay centered if the sensor setup changes. Distances, speeds and angles are parameters, the start pose and the Twist/TwistStamped choice are launch arguments, and the 20 m × 20 m map is much bigger than the 15 ft × 20 ft world.

Mapping and odometry. The mapper works in the odom frame with no SLAM correction, so wheel slip can distort the map. It warns if the pose leaves the grid.

Team contributions. Thomas Kaseca built the Gazebo environment and the first TurtleBot 4 simulation and movement. Kiara Nelson wrote the rest of the robot code. Sadia Tasnim led the documentation and final written integration.

Sources. ROS 2 Jazzy, Gazebo Harmonic, the TurtleBot 4 and Create 3 packages, TF2, NumPy and teleop_twist_keyboard are used as provided. The log-odds update is a standard textbook technique.