# Project 1: Reactive Robotics using ROS 2 and Gazebo

# World and Launch File Documentation

Team: Thomas Kaseca, Kiara Nelson, Sadia Tasnim.

CS 4023/5023

Introduction to Intelligent Robotics

## Overview

Two files set up the simulation. The world file (cust_robo_room.sdf) builds the room and hallway the robot drives in. The launch file (simulation.launch.py) starts everything with one command: the simulator, the TurtleBot 4, and our controller and mapper. Two small helper files, setup.py and package.xml, tell ROS 2 how to install our package, proj1.

## The world file

The world file is written in SDF, the XML format Gazebo reads. It describes one world named project1_world. The name matters, because the robot's lidar connection is built from it, and a wrong name leaves the robot blind.

Basic setup. The file turns on three Gazebo systems: physics so things collide, user commands, and a scene broadcaster so the 3D window can draw the world. It skips the sensor system on purpose, because the TurtleBot brings its own and loading it twice crashes Gazebo. A light called the sun hangs 10 m up and casts shadows, and the floor is a flat 100 m × 100 m ground plane.

Walls. The room and hallway are one fixed model built from seven wall boxes, each 2.5 m tall and 0.15 m thick. Each box has a collision shape (what the bumper touches) and a visual shape (what you see and what the lidar picks up). The walls never move.

Layout. The building's center is (0, 0), with east as +x and north as +y. It measures 15 ft × 20 ft (4.572 m × 6.096 m) and holds:

- The room, 10 ft wide and 15 ft long, in the northwest.

- The long hall, 5 ft wide and 20 ft long, running north to south along the room's east side.

- The short hall, 5 ft wide and 15 ft long, running east to west along the room's south side. The two halls together make the backwards L.

The only way between the room and the hall is a 5 ft doorway in the room's east wall, in the middle of its long side and 5 ft from the north end of the long hall. Both ends of the hall are closed. The straight walls, corners, hallway and doorway give the robot something to test every behavior on. All distances are in meters, and wall positions are measured to the wall centers, so the open floor is about 6 in narrower than the nominal sizes.

## The launch file

The launch file is a Python script that starts everything in the right order. Nearly anything the TA might want to change is a setting (a launch argument), so no file editing is needed.

Settings (default in parentheses):

- world: path to the world file (default: our cust_robo_room.sdf).

- world_name: overrides the world's name (default: read from the file).

- model: standard or lite TurtleBot 4 (standard).

- x, y, z, yaw: starting pose (−0.762 m, 0.762 m, 0, 0 rad), which is the middle of the room facing the doorway.

- rviz: open RViz (false).

- stamped: send TwistStamped instead of Twist commands (true).

- disable_reflexes: turn off the base's own bump reflexes (true).

What happens when you launch, in order:

- Tell Gazebo where the TurtleBot 4 and Create 3 3D models live.

- Start Gazebo with the chosen world, running right away. Closing it ends the whole launch.

- Copy Gazebo's simulated clock into ROS as /clock, so our programs keep time with the simulation.

- Read the world's name from the world file and call the TurtleBot 4 spawn launch. It places the robot and its dock at the start pose and connects Gazebo and ROS for the lidar (/scan), odometry (/odom), driving (/cmd_vel), bumper hits (/hazard_detection) and coordinate frames (/tf).

- Start our controller (reactive_node), which decides how the robot moves.

- Start our mapper (occupancy_mapper), which publishes the map on /map.

- After 5 s, switch off the base's habit of backing away from bumps, which would fight our halt behavior. The script retries for about 2 minutes in case the base is slow to start.

The launch file calls the standard TurtleBot 4 and ros_gz_sim launch files instead of copying them.

## How to run it on the CSN Linux machines

The machines need ROS 2 Jazzy with the TurtleBot 4 simulator, teleop_twist_keyboard and nav2_map_server.

1. One-time setup.

mkdir -p ~/ros2_ws/src && cd ~/ros2_ws/src
git clone https://github.com/seepaa/IRP1

2. In every terminal, before any ros2 command, run these three lines. The lab machines default to ROS_LOCALHOST_ONLY=1, which keeps the map from reaching RViz once the simulator's roughly 45 nodes are running. Use the same ROS_DOMAIN_ID (any number from 1 to 100) in every terminal.

export ROS_LOCALHOST_ONLY=0
export ROS_DOMAIN_ID=42
source /opt/ros/jazzy/setup.bash

3. Terminal 1: build and launch.

cd ~/ros2_ws
colcon build --packages-select proj1
source install/setup.bash
ros2 launch proj1 simulation.launch.py

To test other cases, add settings to the last line, for example x:=0.5 y:=-2.0 yaw:=1.57 or world:=/path/to/other.sdf.

4. Terminal 2: keyboard control (optional). Click in this terminal before pressing keys. Keys go to the controller, not straight to the wheels, so a bump still halts the robot.

ros2 run teleop_twist_keyboard teleop_twist_keyboard --ros-args -p stamped:=true -r cmd_vel:=/teleop_cmd

5. Terminal 3: see the map. Run rviz2. Set Fixed Frame to odom, then choose Add, By topic, /map, Map. If nothing shows, set the Map display's Durability Policy to Transient Local. Gray is unknown, white is free and black is wall.

6. Shut down. Press Ctrl+C in the launch terminal, then clear any leftover simulator. The second command should print nothing, because a leftover simulator slows the next run and freezes the map.

pkill -f "gz sim"
pgrep -af "gz sim"

If the robot ignores commands, relaunch with stamped:=false and drop -p stamped:=true from the teleop command.

Credit: