# Project 2: Reactive Robotics on a Physical TurtleBot 4

# Change Documentation

Team: Sadia Tasnim, Kiara Nelson, Thomas Kaseca

CS 4023/5023 Introduction to Intelligent Robotics

This document lists the changes we made to our code and launch files between Project 1 (Gazebo simulation) and Project 2 (physical TurtleBot 4 in REPF B4), and why we made them.

## Launch files

New robot.launch.py. We kept Project 1’s simulation.launch.py for the simulator and wrote a new launch file for Project 2 instead of editing it.

- Removed: Gazebo, the /clock bridge, robot spawning, Gazebo resource paths, and the world and start-pose arguments. The real robot runs its own drivers and already publishes /scan, /odom, /hazard_detection and /tf, so there is nothing to simulate, spawn or bridge.

- use_sim_time is now false. Without Gazebo nothing publishes /clock, so with sim time the nodes’ clocks would stay at zero. The teleop timeout would then never expire, and one key press would keep control forever.

- Starts the lidar motor (/start_motor) at launch and stops it (/stop_motor) on Ctrl+C. The real lidar doesn’t spin until it is told to, and lab rules require it to be off before the robot is docked.

- Kept the reflex-disable step. The Create 3 turns its bump reflexes back on after every reboot, and they would fight our halt behavior.

- The mapper is now optional (mapper:=false by default). Project 2 doesn’t require mapping.

- New tuning arguments: forward_speed, turn_speed and halt_on_cliff, so the robot can be tuned in the lab without editing code.

## Parameter changes

| Parameter | Project 1 | Project 2 | Reason |
| --- | --- | --- | --- |
| forward_speed | 0.2 m/s | 0.1 m/s | OU lab guide recommends 0.1; safer in a shared room |
| turn_speed | 0.8 rad/s | 0.5 rad/s | Less overshoot on real wheels |
| heading_tolerance_deg | 3 | 5 | Real odometry is noisier; avoids hunting around the target heading |
| min_valid_range | (new) | 0.15 m | Ignore lidar returns from the robot’s own tower posts |
| symmetry_tolerance | 0.15 m | 0.08 m | [reason to confirm with team] |

## Controller (reactive_node.py)

- /odom is subscribed with sensor-data (best-effort) QoS. A reliable subscriber receives nothing from a best-effort publisher, so the controller would never get its first pose and would sit idle without any error.

- No more guessing which way the lidar faces. Project 1 assumed the lidar faced forward when its TF wasn’t available. The real TurtleBot 4’s lidar is mounted rotated, so that guess would swap front and side. Scans are now ignored until the TF arrives.

- The robot waits for both odometry and a usable scan before moving. If the lidar isn’t spinning, the robot stays still instead of driving blind.

- Halt also covers cliff and wheel-drop hazards (halt_on_cliff, on by default). On the physical robot, these hazards can damage it.

- Halt releases if hazard messages stop arriving (hazard_timeout, 0.5 s). Over Wi-Fi, a lost “released” message could otherwise leave the robot halted forever.

- min_valid_range parameter added (see the table above).

- Removed the debug log in escape() that printed the left and right distances and their difference. [reason to confirm with team]

- Removed a second lookup_sensor_transform method inside the controller class; the module-level function of the same name is still used. [reason to confirm with team]

## Mapper (occupancy_mapper.py)

- /odom is subscribed with sensor-data QoS, for the same reason as the controller.

## Package files

- package.xml: added std_srvs (the lidar start/stop service type) and marked the Gazebo packages as needed only for the simulation launch. Updated the package descriptions in package.xml and setup.py.

## Credits

Contributions: [to be filled in]