# Project 2: Reactive Robotics on a Physical TurtleBot 4

# Launch File Documentation

Team:Kiara Nelson, Sadia Tasnim, Thomas Kaseca

CS 4023/5023 Introduction to Intelligent Robotics

## Overview

One launch file, robot.launch.py, runs Project 2 on the physical TurtleBot 4 in REPF B4. It runs on the lab desktop, not on the robot: the robot’s own onboard computer already runs its drivers and publishes /scan, /odom, /hazard_detection and /tf, and accepts velocity commands on /cmd_vel. Nothing is simulated, so unlike Project 1’s simulation.launch.py it starts no Gazebo, world or robot model.

## Settings (launch arguments, so no file editing is needed)

| Setting | Default | What it does |
| --- | --- | --- |
| stamped | true | Publish TwistStamped (true) or Twist on /cmd_vel |
| forward_speed | 0.1 m/s | Straight-ahead driving speed (OU lab guide value) |
| turn_speed | 0.5 rad/s | In-place turn speed for escape, avoid and random turn |
| disable_reflexes | true | Turn off the Create 3's built-in bump reflexes |
| start_lidar | true | Start the lidar motor at launch, stop it on exit |
| halt_on_cliff | true | Halt on cliff / wheel-drop hazards as well as bumper hits |
| mapper | false | Also run the occupancy-grid mapper (publishes /map) |

## What happens when you launch it, in order

- Start the lidar motor (/start_motor service call). The real RPLIDAR does not spin on its own.

- After 2 s, turn off the Create 3’s bump reflexes (retrying for about 2 minutes), so the base does not back away from bumps on its own and fight our halt behavior.

- Start our reactive controller, plus the occupancy mapper only if mapper:=true.

- On Ctrl+C, stop the lidar motor (/stop_motor); the robot must not be docked with the lidar spinning.

## How to run it on the CSN Linux machines

1. One-time setup (same repo as Project 1):

mkdir -p ~/ros2_ws/src && cd ~/ros2_ws/src && git clone https://github.com/seepaa/IRP1
cd ~/ros2_ws && source /opt/ros/jazzy/setup.bash && colcon build --packages-select proj1

2. Check the robot (SSH terminal). Carry it by the base, place it on the floor, then:

ssh student@<robotName>.cs.nor.ou.edu   # password: student
ros2 topic list                          # expect /scan, /tf, /odom

If they are missing, run turtlebot4-daemon-restart, wait for the chime and check again. If docked, undock:

ros2 action send_goal /undock irobot_create_msgs/action/Undock "{}"

3. In every desktop terminal, run robot-setup.sh, enter the robot’s name, run the lines it prints in order, then source ROS and our package. Do not use Project 1’s ROS_LOCALHOST_ONLY=0 / ROS_DOMAIN_ID=42 exports here; they put the terminal on a different network than the robot.

source /opt/ros/jazzy/setup.bash
source ~/ros2_ws/install/setup.bash
ros2 topic list                          # should show the robot’s topics

4. Terminal 1 — launch:

ros2 launch proj1 robot.launch.py

5. Terminal 2 — keyboard control (optional):

ros2 run teleop_twist_keyboard teleop_twist_keyboard \
  --ros-args -p stamped:=true -r cmd_vel:=/teleop_cmd

Each key press holds control for 5 s; lower the teleop speeds (z/c keys) to about 0.1 m/s and 0.2 rad/s.

6. To stop: Ctrl+C the launch (stops the lidar automatically), then return the robot to its charging dock.